The Original Conservation Rooflight for 17.5° - 65.0° pitched roofs.

- Manufacturer: The Rooflight Co.

  - Contact Details

    - Address: 	Unit 1 Bourton Industrial Park

Bourton-on-the-Water

Cheltenham

GL54 2HQ

    - Telephone: +44 (0)1993 833155

    - Website: www.therooflightco.com

    - Email: hello@therooflightco.com

  - Product Reference: The Conservation Rooflight (CR___)

  - Roof pitch:     °

- Product Performance

  - Whole Window U-Value (Uw): 1.4W/m2K to EN ISO 10077-2

  - Air permeability: Class 4 (EN1026)

  - Wind resistance: Class 5 (EN12211)

  - Water resistance: Class E1050 (EN1027)

- Frame

  - Material: Patented welded 3mm steel frame (as per GB251419)

  - Finish

    - Corrosion Protection: EN ISO 12944 to classification C5-M Long.

    - Coating: Polyester Powder Coating minimum dry film thickness 140 µm

    - Colour: RAL 9005 Semi Textured Black

  - Weather seals: Dual, PVC flipper and EPDM bubble seals.

  - Top Hinged: Externally top-hinged using traditional hinge posts with A4 stainless steel fixings.

- Glazing

  - Standards/Type: BSI Kitemarked (KM93754) Insulated glazing units to EN1279-3

  - Construction: 24mm insulated glazing units with stepped edge to outer leaf, comprising:

4mm clear toughened external pane

16mm cavity with bronze spacer, Argon filled to 90%

4mm clear toughened Low E inner pane

  - Centre pane U value (Ug): 1.0 W/m²K

  - Acoustic Performance of Glazing: Rw=31dB (-1; -4) to EN12758

  - External feature sealant fillet: One-part low modulus neutral cure silicone sealant in black finish.

  - Stepped edge unit with cill mounted period glazing clips.

- Operation

  - Operation Type: Brass/Chrome/Brass winder with handle or wheel.

Brass/Chrome winder with out of reach handle

Motorised opening to stroke length 150mm/300mm/450mm

[Delete as Appropriate]

- Accessories: Extension pole and winding crank for out of reach handle [only applicable for manual out of reach opening option. Delete as appropriate]

- Glazing Bars: Structurally bonded metallic glazing bars applied to internal and external faces of the double glazed unit to provide the period aesthetic without compromising the double glazed units performance, complete with feature sealant fillet.

- Third Party Certification: Conformity Marking in accordance with EN 14351-1. BSI Kitemark to glazing ref: KM93754.